# Smart Lift Controller: Verilog Study Guide

Oct 13, 2026 · @Pranav

Build the controller in this order: floor_tracker, request_manager, lift_fsm, then the top module. Write each one yourself from the staged hints, and prove it with the tests here before you move on.

## How to use this guide

Work every module through the same six steps and you will understand it instead of copying it. The steps are the same for all four files, so after the first module the routine is automatic.

- Contract. Read the port table and write only the module ... endmodule header, from memory if you can.

- Hardware picture. Decide which parts are flip-flops (they remember) and which are gates (they react). Sketch it on paper before typing.

- Stage the code. Write it in the small stages given here. Each stage must compile before you start the next.

- Predict. Use the expected-value tables to know what the waveform should look like before you run anything.

- Simulate and compare. A mismatch between prediction and waveform is where the learning happens. Find out which of the two was wrong.

- Compare with the reference last. Only after your version passes, read the reference and compare ideas, not text.

### Build order

| Order | Module | Why here | What it teaches |
| --- | --- | --- | --- |
| 1 | floor_tracker | Smallest module, one register | clock edges, reset, non-blocking assignment |
| 2 | request_manager | Pure storage logic, no control flow | vectors, one-hot decode, set and clear priority |
| 3 | lift_fsm | Needs the other two in your head | state registers, combinational lookahead, outputs |
| 4 | smart_lift_top | Only wiring | instantiation, port connection, fan-out |

### Files and commands

One module per file, and the file name equals the module name. Each module gets a testbench named tb_<module>.v that lives beside it.

iverilog -g2005 -Wall -o sim_tracker tb_floor_tracker.v floor_tracker.v
vvp sim_tracker
gtkwave dump.vcd

Inside every testbench add these two lines at the start of the initial block so GTKWave has something to open:

$dumpfile("dump.vcd");
$dumpvars(0, tb_floor_tracker);   // change the name per testbench

Keep -Wall on. It warns about width mismatches and unconnected ports, which are the two most common silent bugs in this design.

## Verilog foundations

Every bug you will hit in this project comes from one of nine ideas below, so read them before writing the first module and come back when a waveform surprises you.

### 1. A module is a circuit, not a program

Nothing runs top to bottom. Every assign and every always block exists at the same time and reacts whenever its inputs change, like parts soldered onto a board. The order you write blocks in the file does not matter.

### 2. wire and reg

A wire is a connection. It is driven by an assign, by a module output, or by an instance port. A reg is a variable that can be assigned inside an always block. The name misleads: a reg becomes a flip-flop only if you assign it inside a clocked block. A reg assigned inside always @(*) is just a wire made of gates.

### 3. Combinational logic: the answer depends only on the inputs right now

always @(*) begin
    y = 1'b0;            // default first
    if (sel) y = a;      // overrides the default only when sel is high
end

The rule that prevents latches: every signal assigned in the block must get a value on every possible path. Writing a default at the top of the block guarantees it. If you skip it and write only if (sel) y = a;, hardware must remember the old y when sel is low, so the tool builds a latch you never asked for.

### 4. Sequential logic: memory that updates only on a clock edge

always @(posedge clk or negedge rst_n) begin
    if (!rst_n) q <= 1'b0;   // reset wins over everything
    else        q <= d;      // otherwise capture d at the rising edge
end

Between edges q holds its value no matter what d does. The reset here is asynchronous (it acts the instant rst_n falls) and active low, which is what the _n suffix means. Use the same reset style in every module so they come out of reset together.

### 5. Blocking = versus non-blocking <=

| Where | Use | Why |
| --- | --- | --- |
| always @(*) (combinational) | = | Later lines may use the result of earlier lines, like a chain of gates |
| always @(posedge clk) (sequential) | <= | Every flip-flop samples the old values at the same instant, then all update together |

// Swaps a and b on every clock. Both right-hand sides read the OLD values.
always @(posedge clk) begin
    a <= b;
    b <= a;
end

Write that with = and both end up equal to the old b. Never mix the two styles inside one block.

### 6. Vectors, indexing and one-hot

- reg [3:0] m; is four bits, m[0] the lowest. m[f] with a variable f is a legal run-time bit select, and it is how you ask "is floor f requested?".

- One-hot means exactly one bit is 1. You make a one-hot mask with a shift: 4'b0001 << f puts the 1 at position f.

- Reduction operators collapse a vector to one bit: |m is 1 if any bit of m is 1, &m is 1 only if all are.

- {4{1'b0}} repeats 1'b0 four times, giving 4'b0000. It lets the width come from a parameter.

### 7. Width rules

Verilog does not warn you when a result is wider than its destination. It silently drops the top bits. Two habits keep you safe: write sized literals (4'b0001, not 1) and keep the -Wall flag on. A bare 1 is 32 bits wide, which can quietly change how an expression is evaluated.

### 8. State machines in two blocks

A clean FSM has three parts: a state register (sequential), next-state logic (combinational), and output logic (combinational). Name the states with localparam and give every case a default branch so an illegal state value falls back to a safe one.

localparam [1:0] IDLE = 2'd0, MOVE_UP = 2'd1, MOVE_DOWN = 2'd2;
reg [1:0] state, next_state;

### 9. Hierarchy and wiring

A parent module creates a child with an instance and connects each port by name, never by position:

floor_tracker u_tracker (
    .clk(clk), .rst_n(rst_n),
    .move_en(motor_enable), .move_dir(motor_dir),
    .curr_floor(curr_floor)
);

One net can feed several ports (this is called fan-out). An output port of the parent can also be read inside the parent, which is how motor_enable reaches both the chip pin and the floor tracker.

## Decisions to freeze before coding

Your design document leaves twelve choices open, and each one changes either a port width or a line of logic. Settle them now and write them at the top of each file, because changing one later means editing several modules.

| # | Decision | Choice used in this guide | Why | Alternative and its cost |
| --- | --- | --- | --- | --- |
| 1 | Number of floors | 4 (floors 0 to 3), set by a NUM_FLOORS parameter | Floor fields are 2 bits, and 2 bits count to 4. The summary says 5 floors, which a 2-bit field cannot address | 3-bit floor fields for 5 floors; this breaks the 8-in budget |
| 2 | Reset | Asynchronous, active low (rst_n); lift starts IDLE at floor 0 | Same style in all three modules so they leave reset together | Synchronous reset is fine if you use it everywhere |
| 3 | outsider_dir polarity | 1 means up, 0 means down | Matches the LED names | Either, as long as the testbench agrees |
| 4 | Meaning of req_valid | When high, register both the hall call (outsider_loc with outsider_dir) and the car call (insider_dest) | One strobe covers all call fields, and no spare pin says which kind it is | Add a pin (not possible, inputs are full) or decode by convention |
| 5 | When a request is cleared | lift_fsm raises clear_floor for the cycle it is stopped at that floor; the bit clears at the next clock edge | The FSM is the only block that knows it has served the floor | Clear on arrival inside the tracker; mixes datapath and control |
| 6 | Press and clear in the same cycle | The new press wins and stays pending | A real press must never be lost | Clear wins; a press at the floor being served can vanish |
| 7 | Buttons that do not exist | Up at the top floor and down at floor 0 are ignored | The hardware has no such button | Accept them; the request is simply never served wrongly |
| 8 | State meaning | The three states record the direction of travel (IDLE, MOVE_UP, MOVE_DOWN), not whether the motor is running | Lets the lift pause at a floor without losing its direction | 5-state FSM, which your journey document already rejected |
| 9 | Motor and stops | One clock cycle moves one floor. A stop costs one cycle with the motor off and clear_floor high. No door state | Matches the single-cycle floor transitions in the design notes | Add a DOOR state and a timer later (stretch goal) |
| 10 | Tie-break when both sides have requests while idle | Go up first | Simple and deterministic | Nearest-first needs a distance comparison |
| 11 | Merged request mask | One 4-bit mask loses the hall direction | The design notes specify one global bitmask | Keep up and down masks separate; see stretch goals |
| 12 | Output style | motor_dir and both LEDs come straight from the state register; motor_enable also depends on the lookahead | Keeps three outputs glitch-free and one reactive | Register every output (one cycle slower) |

## Signal contract

This table is the single source of truth for every module header and for the top-level wiring. When a module will not connect, check it against this table first.

| Signal | Width | Driven by | Read by | Meaning |
| --- | --- | --- | --- | --- |
| clk | 1 | pin | all three modules | Rising edge is the only moment anything changes |
| rst_n | 1 | pin | all three modules | Asynchronous, active low |
| outsider_loc | 2 | pin | request_manager | Floor of the hall button pressed |
| insider_dest | 2 | pin | request_manager | Floor chosen inside the car |
| outsider_dir | 1 | pin | request_manager | 1 = up call, 0 = down call |
| req_valid | 1 | pin | request_manager | High for one cycle when the call fields hold a real press |
| pending_mask | 4 | request_manager | lift_fsm | Bit i is 1 when floor i has any request |
| clear_floor | 1 | lift_fsm | request_manager | Requests at curr_floor are served: clear them |
| curr_floor | 2 | floor_tracker | lift_fsm, request_manager, pin | The floor the lift is at now |
| motor_enable | 1 | lift_fsm | pin, floor_tracker (move_en) | 1 = the lift moves one floor this cycle |
| motor_dir | 1 | lift_fsm | pin, floor_tracker (move_dir) | 1 = up, 0 = down (only meaningful when enabled) |
| dir_up_led | 1 | lift_fsm | pin | On while the travel direction is up |
| dir_down_led | 1 | lift_fsm | pin | On while the travel direction is down |
| curr_floor_out | 2 | floor_tracker | pin | Same net as curr_floor, sent to the outside world |

In the signal-map picture the tracker inputs were called move_en and move_dir. They are the same wires as motor_enable and motor_dir: the FSM drives one net, and the top module connects it to both the chip pin and the tracker.

### Timing rules the modules rely on

Read these as promises each block makes to the others. If a module breaks one, the whole system drifts by a cycle.

- Everything changes on the rising (pos) edge of clk. Between two edges every register holds its value, so each block sees the others' values from before the edge.

- A press registered at edge E shows up in pending_mask right after E, and lift_fsm first sees it in the cycle that follows.

- If clear_floor is high during a cycle, the matching bits clear at the edge that ends that cycle.

- If motor_enable is high during a cycle, curr_floor changes at the edge that ends that cycle. So lift_fsm always reads a floor that matches the mask it is looking at.

- There is no combinational loop. The feedback paths (FSM to tracker to FSM, and FSM to request_manager to FSM) all pass through flip-flops. If your simulator hangs or shows x forever, you wired a path that skips a register.

## Module 1: floor_tracker

This is the datapath: a 2-bit counter that remembers which floor the lift is at and moves it one step per clock when told to. Start here because it has exactly one register, so every idea in the foundations section shows up in a few lines.

### Contract

| Port | Direction | Width | Meaning |
| --- | --- | --- | --- |
| clk | in | 1 | Rising-edge clock |
| rst_n | in | 1 | Async active-low reset; floor goes to 0 |
| move_en | in | 1 | 1 = change floor at the next edge, 0 = hold |
| move_dir | in | 1 | 1 = up (+1), 0 = down (-1); ignored when move_en is 0 |
| curr_floor | out | 2 | Current floor, 0 to 3 |

### Think in hardware

Picture the floor number sitting in two flip-flops. Their output feeds an adder that makes both floor + 1 and floor - 1. A first multiplexer picks one of them using move_dir. A second multiplexer chooses between that result and the unchanged floor using move_en. Its output goes back to the flip-flops' inputs, and the clock edge decides when the new value is captured. That is all the module is. Notice that there is no input from any request, because the tracker never decides where to go. It only obeys.

### Build it in stages

Stage 1: header. Write the module with the five ports from the table. Ask yourself: curr_floor is assigned inside an always @(posedge clk) block, so what must its type be? (Answer: output reg [1:0]. A plain output wire cannot be assigned in a procedural block and will not compile.)

Stage 2: reset only. Add the clocked block with the async reset and nothing else. Simulate and confirm curr_floor is 0 after reset.

Stage 3: move and hold. Fill the gaps below. Hint: you can write the new value as curr_floor + 2'd1 and curr_floor - 2'd1. In a clocked block, a path that assigns nothing simply keeps the old value, so the "hold" case needs no code. This is the opposite of the combinational rule, where a missing assignment makes a latch.

module floor_tracker (
    input  wire       clk,
    input  wire       rst_n,
    input  wire       move_en,
    input  wire       move_dir,
    output reg  [1:0] curr_floor
);
    always @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            // YOU: put the lift on floor 0
        end else begin
            // YOU: only when move_en is high, step up or down
        end
    end
endmodule

Stage 4: guard the ends. Two bits wrap around: floor 3 plus 1 becomes 0. The FSM should never ask for that, but a bug elsewhere would make the lift teleport. Add a condition so that up is ignored at floor 3 and down is ignored at floor 0. This is called defensive design, and it turns a silent teleport into a lift that simply stops.

### Check yourself

Write the testbench skeleton below, then add one check per row of the table. Drive inputs just after the falling edge and check just after the rising edge, so you never sample while a signal is changing.

`timescale 1ns/1ps
module tb_floor_tracker;
    reg clk = 0, rst_n = 0, move_en = 0, move_dir = 0;
    wire [1:0] curr_floor;

    floor_tracker dut (.clk(clk), .rst_n(rst_n), .move_en(move_en),
                       .move_dir(move_dir), .curr_floor(curr_floor));

    always #5 clk = ~clk;                 // 10 ns period

    task check(input [1:0] expected);
        begin
            if (curr_floor !== expected)
                $display("FAIL t=%0t: floor=%0d expected %0d", $time, curr_floor, expected);
            else
                $display("pass t=%0t: floor=%0d", $time, curr_floor);
        end
    endtask

    initial begin
        $dumpfile("dump.vcd"); $dumpvars(0, tb_floor_tracker);
        // YOU: release reset, then one step per row of the table
        $finish;
    end
endmodule

| Step | Inputs before the edge | Expected curr_floor after the edge |
| --- | --- | --- |
| 1 | rst_n low | 0 (immediately, no clock needed) |
| 2 | reset released, move_en 0 | 0 (holds) |
| 3 | move_en 1, move_dir 1 | 1 |
| 4 | same inputs again | 2 |
| 5 | same inputs again | 3 |
| 6 | same inputs at floor 3 | 3 (guard from stage 4) |
| 7 | move_en 0 | 3 (holds) |
| 8 | move_en 1, move_dir 0 | 2 |
| 9 | same twice more | 1, then 0 |
| 10 | move_dir 0 at floor 0 | 0 (guard) |
| 11 | at floor 2, pull rst_n low between edges | 0 straight away (asynchronous reset) |

### Common mistakes

- Using = instead of <= inside the clocked block. It often still works in a single register, then breaks the moment you add a second.

- Wrong reset polarity: if (rst_n) where you meant if (!rst_n) leaves the lift stuck at floor 0.

- Forgetting negedge rst_n in the sensitivity list. Then the reset becomes synchronous and step 11 fails.

- Declaring the output as wire. The compiler error mentions "not a valid l-value".

### Reference solution

Compare only after your version passes every row above.

module floor_tracker (
    input  wire       clk,
    input  wire       rst_n,
    input  wire       move_en,
    input  wire       move_dir,    // 1 = up, 0 = down
    output reg  [1:0] curr_floor
);
    always @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            curr_floor <= 2'd0;
        end else if (move_en) begin
            if (move_dir) begin
                if (curr_floor != 2'd3) curr_floor <= curr_floor + 2'd1;
            end else begin
                if (curr_floor != 2'd0) curr_floor <= curr_floor - 2'd1;
            end
        end
    end
endmodule

## Module 2: request_manager

This is the memory block. A button press lasts one clock cycle, so something must catch it and keep it until the lift has served that floor. That something is three banks of flip-flops, one bit per floor, plus the logic that sets and clears them. It has no idea where the lift is going; it only remembers and reports.

### Contract

| Port | Direction | Width | Meaning |
| --- | --- | --- | --- |
| clk | in | 1 | Rising-edge clock |
| rst_n | in | 1 | Async active-low reset; all requests cleared |
| outsider_loc | in | 2 | Floor of the hall button |
| insider_dest | in | 2 | Floor chosen inside the car |
| outsider_dir | in | 1 | 1 = up call, 0 = down call |
| req_valid | in | 1 | Strobe: the fields above hold a real press this cycle |
| curr_floor | in | 2 | Which floor the lift is at (from floor_tracker) |
| clear_floor | in | 1 | From lift_fsm: requests at curr_floor are served |
| pending_mask | out | NUM_FLOORS (4) | Bit i is 1 when floor i has any request |

The module has a parameter, NUM_FLOORS, defaulting to 4, so vector widths come from one place.

### Think in hardware

There are 12 flip-flops in three banks of 4: hall_up, hall_down and car_calls. Every one of them follows the same small equation, shown here for bit i of one bank:

next[i] = set[i]  OR  ( q[i] AND NOT clr[i] )

set[i] is 1 when a valid press names floor i. clr[i] is 1 when clear_floor is high and curr_floor equals i. The OR means a press wins over a clear in the same cycle (decision 6). Three small decoders turn the 2-bit floor numbers into one-hot vectors so that set and clr can be built for all four bits at once. Finally pending_mask is an OR gate on the flop outputs. It is not registered, so it changes right after the clock edge that changed a bank.

Why two kinds of block? The next-state equations are combinational (always @(*) with =), and the 12 flip-flops are sequential (always @(posedge clk ...) with <=). Keeping them apart makes the "set beats clear" rule visible as the order of two if statements.

### In plain words: three whiteboards

Think of the module as three whiteboards, each with 4 tick boxes, one per floor.

- hall_up has a tick when someone at that floor pressed "up".

- hall_down has a tick when someone at that floor pressed "down".

- car_calls has a tick when someone inside the lift pressed that floor.

A button press lasts only one clock cycle, so the module's job is to tick a box and keep it ticked until the lift has served that floor. Every stage below is one piece of that.

Stage 1: header. This is just the outline of the box. NUM_FLOORS is how many tick boxes each whiteboard has, and it defaults to 4. pending_mask is a wire because you will drive it with an assign later, not inside an always block.

Stage 2: storage and reset. A reg is memory, so each whiteboard is a reg. Each board also has a "next" copy (hall_up_nxt and so on), which is what the board will look like after the next clock tick. On each clock edge you copy next into now. On reset you wipe all three boards to zero. The test is simple: reset, then check that everything reads 0.

Stage 3: one-hot decoders. A floor arrives as a number, such as 2, but to tick a box you need a stencil with a single 1 in the right position. The stencils for floors 0 to 3 are 0001, 0010, 0100 and 1000. You make one by starting with 0001 and sliding the 1 left by the floor number. You need three stencils: where the hall button was pressed, which floor was picked inside the car, and where the lift is now.

Stage 4: next-state logic. Start from "next = now", so by default nothing changes. Then do two things in this order:

- Clear. If clear_floor is high, erase the current floor's tick on all three boards. Invert the stencil, then AND it with the board. For floor 2, ~0100 is 1011, so every box is kept except box 2.

- Set. If req_valid is high, add ticks by ORing the stencil in. An up call goes on hall_up, a down call on hall_down, and the car floor always goes on car_calls.

The order matters when a press and a clear happen in the same cycle. Say the lift is stopped at floor 2 and someone presses floor 2 at that moment. With clear first and set second, the press is added after the erase, so it survives. If you swap the two, the set happens first and the clear then erases it, so a real press vanishes. Predict that before you simulate, and then check that you were right.

Stage 5: buttons that don't exist. The top floor has no up button, and floor 0 has no down button. So you make two filters:

- ~(one << (NUM_FLOORS-1)) gives 0111, which blocks the top box. Use it on hall_up.

- ~one gives 1110, which blocks box 0. Use it on hall_down.

AND the filter with the stencil before setting. An up press at floor 3 becomes 1000 & 0111 = 0000, so nothing gets ticked.

Stage 6: merge. pending_mask = hall_up | hall_down | car_calls ticks a floor if any of the three boards has it ticked. With &, a floor would count only if all three boards had it. That would almost never happen, so the lift would ignore most requests.

Build and test each stage before starting the next. Most bugs show up at the stage where you introduced them, and they are much harder to find once all six are written. When your version is ready, send it over and I'll review it the same way I did floor_tracker.

### Build it in stages

Stage 1: header. Write the module with #(parameter NUM_FLOORS = 4) and the ports above. pending_mask will be driven by an assign, so declare it output wire.

Stage 2: storage and reset. Declare hall_up, hall_down, car_calls and their _nxt partners as reg [NUM_FLOORS-1:0]. Write the clocked block: on reset all three banks become zero, otherwise each takes its _nxt value. Test that everything is zero after reset.

Stage 3: one-hot decoders. You need loc_onehot, dest_onehot and curr_onehot. The idea: start from a vector with only bit 0 set and shift it left by the floor number. Hint: wire [NUM_FLOORS-1:0] one = {{(NUM_FLOORS-1){1'b0}}, 1'b1}; then one << outsider_loc. Print the three decoders in a tiny test for floors 0 to 3; you should see 0001, 0010, 0100, 1000.

Stage 4: next-state logic. Fill in the gaps. The order of the two if statements is the whole design decision, so predict what happens if you swap them before you simulate.

always @(*) begin
    hall_up_nxt   = hall_up;      // defaults: keep everything
    hall_down_nxt = hall_down;
    car_calls_nxt = car_calls;

    if (clear_floor) begin
        // YOU: clear the bit for curr_onehot in all three banks
    end

    if (req_valid) begin
        // YOU: if outsider_dir, set loc_onehot in hall_up_nxt
        //      otherwise set loc_onehot in hall_down_nxt
        // YOU: set dest_onehot in car_calls_nxt
    end
end

Stage 5: buttons that do not exist. There is no up button on the top floor and no down button on floor 0. Build two masks: ~(one << (NUM_FLOORS-1)) clears the top bit, and ~one clears bit 0. AND each into the matching loc_onehot before you set it.

Stage 6: merge. One line: assign pending_mask = hall_up | hall_down | car_calls;. Confirm you used | and not &. With & a floor would count as requested only if all three banks said so.

### Check yourself

These steps run in order, each lasting one clock with the listed inputs applied. Fields not mentioned are 0. The expected mask is read just after the clock edge. Bit 0 is the rightmost digit.

| Step | Inputs for this clock | Expected pending_mask | What it proves |
| --- | --- | --- | --- |
| 0 | after reset | 0000 | reset clears all banks |
| 1 | req_valid 1, loc 2, dir 0, dest 2 | 0100 | a press is latched |
| 2 | req_valid 0, loc 1, dest 1 | 0100 | the strobe gates the inputs |
| 3 | req_valid 1, loc 0, dir 1, dest 0 | 0101 | requests accumulate |
| 4 | req_valid 1, loc 3, dir 0, dest 3 | 1101 | the top bit works |
| 5 | clear_floor 1, curr_floor 2 | 1001 | clear removes only the current floor |
| 6 | clear_floor 1, curr_floor 3, and a press at floor 3 (req_valid 1, loc 3, dir 0, dest 3) | 1001 | set beats clear |
| 7 | clear_floor 1, curr_floor 3 | 0001 | clear works on its own |
| 8 | req_valid 1, loc 3, dir 1, dest 0 | 0001 | up call at the top floor is ignored |
| 9 | clear_floor 1, curr_floor 0 | 0000 | clearing floor 0 |
| 10 | req_valid 1, loc 0, dir 0, dest 1 | 0010 | down call at floor 0 is ignored; the car call to floor 1 still lands |
| 11 | pull rst_n low between edges | 0000 | asynchronous reset |

In steps 1, 3, 4 and 6 the testbench drives insider_dest equal to outsider_loc. That is a deliberate trick: because req_valid registers both fields at once (decision 4), driving them to the same floor produces one clean request at that floor. Step 8 and step 10 use different values on purpose, to show what each field does.

One more test worth adding: press loc 1 with dir 1 and dest 1, then clear floor 1 once. Both the hall_up bit and the car_calls bit are set, and one clear must remove both.

### Common mistakes

- Leaving out the default copies at the top of the combinational block. The _nxt signals become latches and the simulation shows strange held values.

- Putting the req_valid block before the clear_floor block. It compiles and runs, and fails only step 6.

- Clearing with curr_floor instead of curr_onehot. The wide-versus-narrow mismatch usually compiles with a warning and clears the wrong bits.

- Resetting only one bank. Test step 0 passes after power-up in some simulators and fails in others, depending on initial x handling.

- Expecting pending_mask to wait for the next edge. It is combinational from the flops, so it updates immediately after the edge that changed them.

### Reference solution

The file request_manager.v delivered earlier in this conversation is the same design with long comments. This is the compact version for comparison.

module request_manager #(parameter NUM_FLOORS = 4) (
    input  wire                  clk,
    input  wire                  rst_n,
    input  wire [1:0]            outsider_loc,
    input  wire [1:0]            insider_dest,
    input  wire                  outsider_dir,
    input  wire                  req_valid,
    input  wire [1:0]            curr_floor,
    input  wire                  clear_floor,
    output wire [NUM_FLOORS-1:0] pending_mask
);
    reg [NUM_FLOORS-1:0] hall_up, hall_down, car_calls;
    reg [NUM_FLOORS-1:0] hall_up_nxt, hall_down_nxt, car_calls_nxt;

    wire [NUM_FLOORS-1:0] one         = {{(NUM_FLOORS-1){1'b0}}, 1'b1};
    wire [NUM_FLOORS-1:0] loc_onehot  = one << outsider_loc;
    wire [NUM_FLOORS-1:0] dest_onehot = one << insider_dest;
    wire [NUM_FLOORS-1:0] curr_onehot = one << curr_floor;
    wire [NUM_FLOORS-1:0] up_ok       = ~(one << (NUM_FLOORS-1));
    wire [NUM_FLOORS-1:0] down_ok     = ~one;

    always @(*) begin
        hall_up_nxt   = hall_up;
        hall_down_nxt = hall_down;
        car_calls_nxt = car_calls;
        if (clear_floor) begin
            hall_up_nxt   = hall_up_nxt   & ~curr_onehot;
            hall_down_nxt = hall_down_nxt & ~curr_onehot;
            car_calls_nxt = car_calls_nxt & ~curr_onehot;
        end
        if (req_valid) begin
            if (outsider_dir) hall_up_nxt   = hall_up_nxt   | (loc_onehot & up_ok);
            else              hall_down_nxt = hall_down_nxt | (loc_onehot & down_ok);
            car_calls_nxt = car_calls_nxt | dest_onehot;
        end
    end

    always @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            hall_up <= 0; hall_down <= 0; car_calls <= 0;
        end else begin
            hall_up <= hall_up_nxt; hall_down <= hall_down_nxt; car_calls <= car_calls_nxt;
        end
    end

    assign pending_mask = hall_up | hall_down | car_calls;
endmodule

## Module 3: lift_fsm

This is the control unit, the only block that makes decisions. Each cycle it compares the pending requests with the floor the lift is at, decides whether to keep going, reverse or stop, and drives the motor, the direction LEDs and the signal that clears a served floor. It has three states and three one-bit questions it asks of the mask. Everything else in the design exists to feed those questions.

### Contract

| Port | Direction | Width | Meaning |
| --- | --- | --- | --- |
| clk | in | 1 | Rising-edge clock |
| rst_n | in | 1 | Async active-low reset; state becomes IDLE |
| pending_mask | in | 4 | Bit i is 1 when floor i has a request |
| curr_floor | in | 2 | The floor the lift is at |
| motor_enable | out | 1 | 1 = move one floor this cycle |
| motor_dir | out | 1 | 1 = up, 0 = down |
| dir_up_led | out | 1 | Travel direction is up |
| dir_down_led | out | 1 | Travel direction is down |
| clear_floor | out | 1 | Serve and clear the requests at curr_floor |

### The three questions (lookahead)

Every decision comes from three combinational signals computed from pending_mask and curr_floor. Call the floor f.

| Signal | Meaning | How to compute it |
| --- | --- | --- |
| stop_here | A request exists at floor f | pending_mask[curr_floor] |
| req_above | A request exists above f | ¦(pending_mask >> (curr_floor + 3'd1)) |
| req_below | A request exists below f | ¦(pending_mask & ((4'b0001 << curr_floor) - 4'b0001)) |

Why these work. Shifting the mask right by f + 1 throws away floors 0 to f, so anything left over is above. For f = 3 the shift is 4, which empties a 4-bit mask, so nothing is above the top floor. For the second one, 4'b0001 << f is a single 1 at position f, and subtracting 1 turns it into ones at every position below f (f = 0 gives 0000, f = 3 gives 0111). ANDing with the mask keeps only requests below. The leading | is the reduction OR from the foundations section, which collapses a vector to one bit.

Use these examples to check your three expressions before building anything else.

| pending_mask | curr_floor | stop_here | req_above | req_below |
| --- | --- | --- | --- | --- |
| 0000 | any | 0 | 0 | 0 |
| 0100 | 0 | 0 | 1 | 0 |
| 0100 | 2 | 1 | 0 | 0 |
| 0100 | 3 | 0 | 0 | 1 |
| 1010 | 1 | 1 | 1 | 0 |
| 1001 | 0 | 1 | 1 | 0 |
| 1001 | 3 | 1 | 0 | 1 |

### Think in hardware

There are two flip-flops holding the state. Around them sit three pieces of combinational logic: the lookahead block above, a next-state block that reads the state and the lookahead flags, and an output block. dir_up_led, dir_down_led and motor_dir depend only on the state, so they change cleanly at clock edges. motor_enable and clear_floor also depend on the lookahead flags, so they react as soon as the mask or the floor changes.

The states mean the direction the lift is travelling, not whether the motor is on (decision 8). That is why the lift can stop at a floor for one cycle, with the motor off, and then carry on in the same direction without a separate state for it.

### The rules

Next state (the first matching row wins):

| Current state | Condition | Next state |
| --- | --- | --- |
| IDLE | req_above | MOVE_UP |
| IDLE | req_below | MOVE_DOWN |
| IDLE | otherwise | IDLE |
| MOVE_UP | req_above | MOVE_UP |
| MOVE_UP | req_below | MOVE_DOWN (reverse) |
| MOVE_UP | otherwise | IDLE |
| MOVE_DOWN | req_below | MOVE_DOWN |
| MOVE_DOWN | req_above | MOVE_UP (reverse) |
| MOVE_DOWN | otherwise | IDLE |

Outputs:

| Output | Rule |
| --- | --- |
| clear_floor | stop_here (in every state, including IDLE) |
| motor_dir | state is MOVE_UP |
| dir_up_led | state is MOVE_UP |
| dir_down_led | state is MOVE_DOWN |
| motor_enable | not stop_here, and ((state is MOVE_UP and req_above) or (state is MOVE_DOWN and req_below)) |

Notice that stop_here does not appear in the next-state table. Stopping at a floor only turns the motor off for that cycle and raises clear_floor. The direction memory is untouched. That single design choice is what makes the "sweep" work: the lift keeps its direction until nothing is left that way.

### Build it in stages

Stage 1: header and states. Write the ports, the three localparam state codes and reg [1:0] state, next_state;.

Stage 2: state register. One clocked block: on reset state <= IDLE, otherwise state <= next_state. Nothing else goes in this block.

Stage 3: lookahead. Three assign statements for the three questions. Check them against the example table above using a throwaway testbench that prints the three flags for each row.

Stage 4: next-state logic. Translate the next-state table. Start with the default next_state = state; so every path assigns something.

always @(*) begin
    next_state = state;
    case (state)
        IDLE: begin
            // YOU
        end
        MOVE_UP: begin
            // YOU
        end
        MOVE_DOWN: begin
            // YOU
        end
        default: next_state = IDLE;
    endcase
end

Stage 5: outputs. Five assign lines, one per row of the output table. The motor_enable expression is the only awkward one. Say it aloud first: "moving up and something above, or moving down and something below, but not while stopping".

### Check yourself

First test the FSM alone. In the testbench, control curr_floor and pending_mask yourself, with no tracker and no request manager attached. Hold the inputs for one clock per row and check the outputs during that cycle and the state after the edge. Sequence 1 covers start, travel, stop, and reverse. It includes jumps of curr_floor that a real tracker would do one step at a time, which is fine for a unit test.

| Cycle | curr_floor | pending_mask | State before edge | motor_enable | motor_dir | clear_floor | LEDs (up, down) | State after edge |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0000 | IDLE | 0 | 0 | 0 | 0, 0 | IDLE |
| 2 | 0 | 0100 | IDLE | 0 | 0 | 0 | 0, 0 | MOVE_UP |
| 3 | 0 | 0100 | MOVE_UP | 1 | 1 | 0 | 1, 0 | MOVE_UP |
| 4 | 2 | 0100 | MOVE_UP | 0 | 1 | 1 | 1, 0 | IDLE |
| 5 | 2 | 0000 | IDLE | 0 | 0 | 0 | 0, 0 | IDLE |
| 6 | 2 | 0001 | IDLE | 0 | 0 | 0 | 0, 0 | MOVE_DOWN |
| 7 | 2 | 0001 | MOVE_DOWN | 1 | 0 | 0 | 0, 1 | MOVE_DOWN |
| 8 | 0 | 0001 | MOVE_DOWN | 0 | 0 | 1 | 0, 1 | IDLE |

Sequence 2 tests stopping in the middle of a sweep and carrying on. Reset first so the state is IDLE.

| Cycle | curr_floor | pending_mask | State before edge | motor_enable | motor_dir | clear_floor | LEDs (up, down) | State after edge |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1010 | IDLE | 0 | 0 | 0 | 0, 0 | MOVE_UP |
| 2 | 1 | 1010 | MOVE_UP | 0 | 1 | 1 | 1, 0 | MOVE_UP |
| 3 | 1 | 1000 | MOVE_UP | 1 | 1 | 0 | 1, 0 | MOVE_UP |
| 4 | 3 | 1000 | MOVE_UP | 0 | 1 | 1 | 1, 0 | IDLE |

In cycle 3 the testbench has already cleared bit 1, playing the part of the request manager. Cycle 2 is the important row: the lift stops at floor 1 with the motor off, but it stays in MOVE_UP because floor 3 is still waiting.

### Common mistakes

- Not gating motor_enable with stop_here. The lift then drives past a floor it is clearing, so the clear lands on the wrong floor one cycle later.

- Asserting clear_floor only in the MOVE states. A request at the floor where an idle lift is standing is then never served.

- Writing the next-state block without the default next_state = state;, or a case without default. Both create latches.

- Mixing the register and the next-state logic into one clocked block. It can work, but the outputs then move one cycle late and every table above is off by one.

- Using curr_floor + 1 unsized in the shift. It often works by luck; the sized 3'd1 removes the doubt.

- Making motor_dir depend on motor_enable. Its value only matters while the motor is on, and tying them together only adds glitches.

### Reference solution

Compare only after your own version passes both sequences.

module lift_fsm (
    input  wire       clk,
    input  wire       rst_n,
    input  wire [3:0] pending_mask,
    input  wire [1:0] curr_floor,
    output wire       motor_enable,
    output wire       motor_dir,
    output wire       dir_up_led,
    output wire       dir_down_led,
    output wire       clear_floor
);
    localparam [1:0] IDLE = 2'd0, MOVE_UP = 2'd1, MOVE_DOWN = 2'd2;
    reg [1:0] state, next_state;

    // lookahead: three questions about the mask
    wire stop_here = pending_mask[curr_floor];
    wire req_above = |(pending_mask >> (curr_floor + 3'd1));
    wire req_below = |(pending_mask & ((4'b0001 << curr_floor) - 4'b0001));

    // state register
    always @(posedge clk or negedge rst_n) begin
        if (!rst_n) state <= IDLE;
        else        state <= next_state;
    end

    // next-state logic
    always @(*) begin
        next_state = state;
        case (state)
            IDLE: begin
                if      (req_above) next_state = MOVE_UP;
                else if (req_below) next_state = MOVE_DOWN;
            end
            MOVE_UP: begin
                if      (req_above) next_state = MOVE_UP;
                else if (req_below) next_state = MOVE_DOWN;
                else                next_state = IDLE;
            end
            MOVE_DOWN: begin
                if      (req_below) next_state = MOVE_DOWN;
                else if (req_above) next_state = MOVE_UP;
                else                next_state = IDLE;
            end
            default: next_state = IDLE;
        endcase
    end

    // outputs
    assign clear_floor  = stop_here;
    assign motor_dir    = (state == MOVE_UP);
    assign dir_up_led   = (state == MOVE_UP);
    assign dir_down_led = (state == MOVE_DOWN);
    assign motor_enable = ~stop_here &
                          ((state == MOVE_UP   & req_above) |
                           (state == MOVE_DOWN & req_below));
endmodule

## Top module: smart_lift_top

The top module contains no logic at all. Its whole job is to create the three blocks and connect every port to the right wire, so a single mistyped name or swapped port can break a system whose parts all passed their own tests. That makes it the place where careful reading of the signal contract pays off.

### Contract

These are exactly the 8 input pins and 6 output pins from your design notes.

| Port | Direction | Width |
| --- | --- | --- |
| clk | in | 1 |
| rst_n | in | 1 |
| outsider_loc | in | 2 |
| insider_dest | in | 2 |
| outsider_dir | in | 1 |
| req_valid | in | 1 |
| motor_enable | out | 1 |
| motor_dir | out | 1 |
| curr_floor_out | out | 2 |
| dir_up_led | out | 1 |
| dir_down_led | out | 1 |

Inside the top you only need to declare three internal wires, because the other nets already exist as ports: pending_mask (4 bits), clear_floor (1 bit) and curr_floor (2 bits). motor_enable and motor_dir are output ports, but an output port can be read inside its own module, so you connect them straight to the tracker.

### Think in hardware

Think of a printed circuit board. The three chips are already built and tested. The top module is the copper traces between their pins, plus the edge connector that goes to the outside world. A trace that connects the wrong pins, or that is missing, is a bug no individual chip test can catch.

### Connection map

Use this table as your checklist. Every row is one named connection in the code.

| Instance | Port | Connects to |
| --- | --- | --- |
| u_tracker (floor_tracker) | clk, rst_n | top-level clk, rst_n |
|  | move_en | motor_enable |
|  | move_dir | motor_dir |
|  | curr_floor | curr_floor |
| u_requests (request_manager) | clk, rst_n | top-level clk, rst_n |
|  | outsider_loc, insider_dest, outsider_dir, req_valid | the matching top-level inputs |
|  | curr_floor | curr_floor |
|  | clear_floor | clear_floor |
|  | pending_mask | pending_mask |
| u_fsm (lift_fsm) | clk, rst_n | top-level clk, rst_n |
|  | pending_mask | pending_mask |
|  | curr_floor | curr_floor |
|  | motor_enable, motor_dir, dir_up_led, dir_down_led | the matching top-level outputs |
|  | clear_floor | clear_floor |
| top level | curr_floor_out | curr_floor through an assign |

### Build it in stages

Stage 1: header. Write the module with the 8 inputs and 6 outputs. Put `default_nettype none on the line before module and `default_nettype wire after endmodule. With it on, a mistyped wire name becomes a compile error. Without it, Verilog quietly invents a new 1-bit wire, which is the classic cause of a lift that never moves.

Stage 2: internal wires. Declare the three wires with the widths above.

Stage 3: instantiate one block at a time. Add floor_tracker, compile and fix. Then request_manager, compile and fix. Then lift_fsm. Always use named connections like .move_en(motor_enable), never positional ones, because a swapped pair of same-width signals compiles without complaint.

Stage 4: the output copy. One line: assign curr_floor_out = curr_floor;.

`default_nettype none
module smart_lift_top (
    // YOU: the 8 inputs and 6 outputs from the contract
);
    // YOU: wire [3:0] pending_mask;  wire clear_floor;  wire [1:0] curr_floor;

    floor_tracker u_tracker (
        // YOU: five connections
    );

    request_manager #(.NUM_FLOORS(4)) u_requests (
        // YOU: nine connections
    );

    lift_fsm u_fsm (
        // YOU: nine connections
    );

    // YOU: curr_floor_out
endmodule
`default_nettype wire

### Check yourself

Compile all five files together, with the warning flag on. Any warning from this command is a real finding, so fix every one:

iverilog -g2005 -Wall -o sim_top tb_smart_lift_top.v smart_lift_top.v \
         floor_tracker.v request_manager.v lift_fsm.v
vvp sim_top

Then run the smoke test: hold rst_n low for a few clocks and release it. Expect curr_floor_out = 0, motor_enable = 0 and both LEDs off, with no x or z anywhere on the outputs. The scenarios in the next section then exercise the whole system.

### Common mistakes

- Leaving clear_floor unconnected. Requests are never cleared, so the lift serves floor 2, sees floor 2 still pending, and sits there forever.

- Connecting curr_floor_out to the tracker but not feeding the same net to the request manager and the FSM. Both then float at z and the outputs turn x.

- Declaring curr_floor as 1 bit by accident (or letting a typo do it). The simulator truncates silently.

- Connecting two blocks' outputs to the same wire. Only lift_fsm may drive motor_enable.

- Passing the wrong width to the NUM_FLOORS parameter. lift_fsm has the width 4 written in, so the parameter must stay 4 or lift_fsm needs to be generalised too.

### Reference solution

`default_nettype none
module smart_lift_top (
    input  wire       clk,
    input  wire       rst_n,
    input  wire [1:0] outsider_loc,
    input  wire [1:0] insider_dest,
    input  wire       outsider_dir,
    input  wire       req_valid,
    output wire       motor_enable,
    output wire       motor_dir,
    output wire [1:0] curr_floor_out,
    output wire       dir_up_led,
    output wire       dir_down_led
);
    wire [3:0] pending_mask;
    wire       clear_floor;
    wire [1:0] curr_floor;

    floor_tracker u_tracker (
        .clk(clk), .rst_n(rst_n),
        .move_en(motor_enable), .move_dir(motor_dir),
        .curr_floor(curr_floor)
    );

    request_manager #(.NUM_FLOORS(4)) u_requests (
        .clk(clk), .rst_n(rst_n),
        .outsider_loc(outsider_loc), .insider_dest(insider_dest),
        .outsider_dir(outsider_dir), .req_valid(req_valid),
        .curr_floor(curr_floor), .clear_floor(clear_floor),
        .pending_mask(pending_mask)
    );

    lift_fsm u_fsm (
        .clk(clk), .rst_n(rst_n),
        .pending_mask(pending_mask), .curr_floor(curr_floor),
        .motor_enable(motor_enable), .motor_dir(motor_dir),
        .dir_up_led(dir_up_led), .dir_down_led(dir_down_led),
        .clear_floor(clear_floor)
    );

    assign curr_floor_out = curr_floor;
endmodule
`default_nettype wire

## Full-system walk-throughs

Once the top module compiles, run these four scenarios and compare your waveform to the tables line by line. They were worked out by hand from the rules in this guide, so a mismatch means your code differs from the design, not that the table is wrong. Each scenario starts from reset, except where it says to continue from the one before.

### How to drive and read the testbench

A press lasts exactly one clock edge. Set the inputs just after a falling edge and clear them on the next falling edge, so the rising edge between them samples a stable value. The task below presses a call at a floor, driving insider_dest to the same floor as outsider_loc so that one press makes one clean request (decision 4).

task press(input [1:0] floor, input dir);
    begin
        @(negedge clk);
        outsider_loc = floor; insider_dest = floor;
        outsider_dir = dir;   req_valid = 1;
        @(negedge clk);
        req_valid = 0;
    end
endtask

// One line per clock, printed mid-cycle when everything has settled
always @(negedge clk)
    $display("%4t | f=%0d mask=%b en=%b dir=%b clr=%b up=%b dn=%b",
             $time, curr_floor_out, dut.pending_mask, motor_enable,
             motor_dir, dut.clear_floor, dir_up_led, dir_down_led);

In the tables, cycle k means the interval after rising edge k and before edge k+1, which is when your $display line is printed. Cycle 0 is just before the press is sampled. The dut.pending_mask and dut.clear_floor names are hierarchical references into the top module's internal wires, which is allowed in a testbench.

### Scenario A: one car call from floor 0 to floor 2

Apply press(2, 0) once, with the lift idle at floor 0. This tests start, travel, stop and return to idle.

| Cycle | State | curr_floor_out | pending_mask | motor_enable | motor_dir | clear_floor | LEDs (up, down) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | IDLE | 0 | 0000 | 0 | 0 | 0 | 0, 0 |
| 1 | IDLE | 0 | 0100 | 0 | 0 | 0 | 0, 0 |
| 2 | MOVE_UP | 0 | 0100 | 1 | 1 | 0 | 1, 0 |
| 3 | MOVE_UP | 1 | 0100 | 1 | 1 | 0 | 1, 0 |
| 4 | MOVE_UP | 2 | 0100 | 0 | 1 | 1 | 1, 0 |
| 5 | IDLE | 2 | 0000 | 0 | 0 | 0 | 0, 0 |

Things to notice. The press is stored after edge 1, yet the lift does not move until cycle 2, because the FSM spends cycle 1 deciding. In cycle 4 the lift has arrived: motor_enable is 0 and clear_floor is 1, but motor_dir and the up LED are still 1 because the state remembers the direction. In cycle 5 the mask is empty, which proves the clear reached the request manager.

### Scenario B: a sweep with a stop in the middle

Starting at floor 0, call press(1, 1) and then, straight after it, press(3, 0). The second press is sampled at edge 3, because the task leaves one idle clock between presses. This tests the sweep: the lift stops at floor 1, keeps its direction, and carries on to floor 3.

| Cycle | State | curr_floor_out | pending_mask | motor_enable | motor_dir | clear_floor | LEDs (up, down) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | IDLE | 0 | 0010 | 0 | 0 | 0 | 0, 0 |
| 2 | MOVE_UP | 0 | 0010 | 1 | 1 | 0 | 1, 0 |
| 3 | MOVE_UP | 1 | 1010 | 0 | 1 | 1 | 1, 0 |
| 4 | MOVE_UP | 1 | 1000 | 1 | 1 | 0 | 1, 0 |
| 5 | MOVE_UP | 2 | 1000 | 1 | 1 | 0 | 1, 0 |
| 6 | MOVE_UP | 3 | 1000 | 0 | 1 | 1 | 1, 0 |
| 7 | IDLE | 3 | 0000 | 0 | 0 | 0 | 0, 0 |

The key row is cycle 3. The lift is at floor 1 with a request there, so it stops with the motor off and clears the floor. The state is still MOVE_UP, so in cycle 4 it sets off again without going through IDLE.

### Scenario C: reversal

Continue from scenario B, with the lift idle at floor 3. Call press(0, 1). This tests the downward direction and the IDLE-to-MOVE_DOWN path.

| Cycle | State | curr_floor_out | pending_mask | motor_enable | motor_dir | clear_floor | LEDs (up, down) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | IDLE | 3 | 0001 | 0 | 0 | 0 | 0, 0 |
| 2 | MOVE_DOWN | 3 | 0001 | 1 | 0 | 0 | 0, 1 |
| 3 | MOVE_DOWN | 2 | 0001 | 1 | 0 | 0 | 0, 1 |
| 4 | MOVE_DOWN | 1 | 0001 | 1 | 0 | 0 | 0, 1 |
| 5 | MOVE_DOWN | 0 | 0001 | 0 | 0 | 1 | 0, 1 |
| 6 | IDLE | 0 | 0000 | 0 | 0 | 0 | 0, 0 |

### Scenario D: a call at the floor where the lift already stands

Continue from scenario C, lift idle at floor 0, then move it to floor 2 by repeating scenario A. With the lift idle at floor 2, call press(2, 0). This tests that a request on the current floor is served without any movement.

| Cycle | State | curr_floor_out | pending_mask | motor_enable | clear_floor |
| --- | --- | --- | --- | --- | --- |
| 1 | IDLE | 2 | 0100 | 0 | 1 |
| 2 | IDLE | 2 | 0000 | 0 | 0 |

### A scenario to design yourself

Work out the table for this one before simulating, then compare. The lift is idle at floor 0. Call press(3, 0). As soon as the lift reaches floor 2, call press(0, 1). Where does the lift go after floor 3, and why does it not turn back at floor 2? (Hint: it keeps its direction while anything is left above, and the request behind it waits.)

## Debugging, limits and next steps

When a waveform disagrees with a table, find the first cycle where they differ, and look at the signal that changed in the cycle before it. That signal is almost always where the bug lives.

### Debug checklist

| Symptom | Most likely cause | Where to look |
| --- | --- | --- |
| Outputs are x after reset | A register with no reset, or an unconnected input floating at z | Reset branch of every clocked block; the connection map in the top section |
| The lift never moves | pending_mask stays 0000 (a press shorter than a clock edge, or req_valid never high at an edge), or motor_enable gated wrongly | Print dut.pending_mask first, then the three lookahead flags |
| The lift reaches a floor and never leaves | clear_floor not connected, or the request manager clears with the wrong floor | Top-level connection of clear_floor; curr_onehot in request_manager |
| Stops one floor early or late | An output registered by mistake, so everything shifts one cycle | Compare the cycle numbers in scenario A row by row |
| Goes past floor 3, or jumps from 3 to 0 | Missing end guard in the tracker, or a wrong req_above shift at the top floor | Stage 4 of floor_tracker; the lookahead example table |
| A press is ignored | The testbench changed inputs exactly at the rising edge (a race) | Drive on the falling edge, as the press task does |
| Goes the wrong way when both sides have requests | Rows of the next-state table in the wrong priority order | The IDLE rows: up is checked first |
| Simulation never finishes | A combinational loop, or no $finish | A path from an output back to its own input that skips a flip-flop |
| One-cycle spikes on motor_enable | Not a bug: it is combinational and reacts to the mask | Decision 12 |

### Invariants worth checking in every simulation

An invariant is a statement that must be true on every clock edge, whatever the test. Putting them in the testbench catches bugs that your hand-picked scenarios miss. Add them as an always @(posedge clk) block that prints a message when one is broken.

| Invariant | Why it must hold |
| --- | --- |
| motor_enable and clear_floor are never both 1 | A lift cannot move and serve a floor in the same cycle |
| dir_up_led and dir_down_led are never both 1 | One direction at a time |
| curr_floor changes by at most 1 per clock | Single-cycle floor transitions |
| curr_floor only changes after a cycle with motor_enable high | The tracker obeys the FSM and nobody else |
| After one lone press, pending_mask is 0000 again within 8 cycles | The lift always finishes what it starts |

reg [1:0] prev_floor;
always @(posedge clk) if (rst_n) begin
    if (motor_enable && dut.clear_floor)
        $display("BUG t=%0t: moving while clearing", $time);
    if (dir_up_led && dir_down_led)
        $display("BUG t=%0t: both direction LEDs on", $time);
    prev_floor <= curr_floor_out;
end

The last two invariants need a little more code (a copy of the previous floor, and a counter). They make good exercises.

### Known limitations of this design

- The merged mask loses hall direction. An up-going lift will stop for a down call at a floor it passes. A true directional sweep keeps hall_up, hall_down and car_calls separate all the way into the FSM.

- req_valid registers both call fields. The outside logic must drive them consistently, which is a workaround for having no pin left to say "hall call" or "car call".

- Four floors only, and lift_fsm has the width written in. The request manager is parameterised, the FSM and tracker are not.

- No dwell time. A stop lasts exactly one clock. A real lift needs doors.

- motor_enable is combinational. In real silicon it could glitch for a moment while the inputs settle. Registering it removes that, at the cost of one cycle of latency.

### Stretch goals, easiest first

- Parameterise floor_tracker and lift_fsm with NUM_FLOORS, using $clog2(NUM_FLOORS) for the floor width.

- Add a self-checking testbench that stores the scenario tables as arrays and compares every cycle.

- Pick the nearest request first when idle instead of always going up.

- Add a DOOR state with a 3-cycle countdown. It needs a new state, a small counter and the motor held off while counting.

- Register all outputs and update every table by one cycle.

- Fix limitation 1: pass the three banks to the FSM, and make the lift stop only for a car call, a hall call in its own direction, or the furthest hall call in the opposite direction at the turnaround.

When you have your own versions of all four files passing the scenarios, send them to me and I will review them against the contract and the invariants.